ME350

Winter 2025 Semester

FINAL REPORT

Team XX

Team Member 1 Name

Team Member 2 Name

Team Member 3 Name

Team Member 4 Name

GSI: GSI Name

<insert a screenshot from CAD of your design here>

“We have fully abided by the University of Michigan College of Engineering Honor Code”

____________________________

____________________________

____________________________

____________________________

Introduction (Reference Only)

Audience (Reference Only)

Table & Figure Formatting (Reference Only)

Report Sections (Reference Only)

Due with DR 1 (Sections 1-3, Appendix A):

Section 1: Design Goals, Requirements, and Specifications (0.5-1pg)

Section 2: Design Selection (1-2 pg)

Section 3: Final Linkage Design and Analysis (2-4 pgs)

Appendix A: Assembly Manual

Due with DR2 (Sections 1-8, Appendices A,B, and C):

Section 4: Motion Generator Revision (0.25-0.5 pg):

Section 5: Energy Conversion Introduction (0.25-0.5 pg):

Section 6: Transmission Ratio and Type Determination (1.0 pg):

Section 7: Final Transmission Design (1 pg) :

Section 8: Torque Transfer Analysis (1-1.5 pg):

Appendix A: Assembly Manual

Appendix B: Bill of Materials for the Final Design

Appendix C: Approval Packages

Due with Final Report (Sections 1-12, Appendices A,B, C, and D):

Section 9: Motor Controls (0.5-1 pg)

Section 10: Encoder Counts, Proximity Sensor Thresholds, and Controller Gains (1-1.5 pg):

Section 11: Final Testing Results/Discussion (1-1.5pg)(Due with Final Report):

Section 12: Design Critique & Evaluation (1-1.5 pg)(Due with Final Report):

Appendix A: Assembly Manual

Appendix B: Bill of Materials for the Final Design

Appendix C: Approval Packages

Appendix D: Wiring Diagram, Arduino Code Changes

Please remove all extraneous text (red text, unassigned sections, page length expectations, prompts, etc) when submitting DR1, DR2, and the Final Report.

Introduction (Reference Only)

This is a template for your team to use when writing your design reports.  The goal here is to write a clear, concise, and informative report that explains the project, the work that has been completed, and the work remaining to be completed.

Audience (Reference Only)

The audience of the written report is a technically adept engineer who has access to the project description. This report should serve as technical documentation for the mechanism you have designed and tested. Feel free to use ME 350 terms, but only those officially documented or introduced in course materials such as the Project Description and Lab Tutorials.

Table & Figure Formatting (Reference Only)

Below are a few rules for formatting tables and figures. Examples are also given below.

- Tables and Figures should have two different increments in the report.

- Tables in Figures in Section/Appendix X should be named as such:

  - Table X.1, Table X.2, etc.

  - Figure X.1, Figure X.2, etc.

- Figures should have a caption that highlights the main conclusion of the figure

  - All figures must be introduced in your text

- Tables should have a title that highlights the main conclusion of the table

  - All tables must be introduced in your text

Figure 2: The max power consumption determined through ADAMS is 0.023 lbf-in/s and the min is -0.025 lbf-in/s; the sudden change to negative power reflects the change in direction of linkage.

Table 6.1: Summary of Project Requirements and Final Results, which shows that all requirements are met. (IN YOUR TEAM’S REPORT, REPLACE THIS WITH THE PROJECT REQUIREMENTS THAT WILL BE PUBLISHED IN THE PROJECT DESCRIPTION.)

| Design Parameter | Target | Min | Max | Predicted Value | Final Testing Value |
| --- | --- | --- | --- | --- | --- |
| Performance Score | Maximize | 0% | 100% | 85% | 87% |
| ABC | Maximize | 0 | 50 | 35 | 24 |

Report Sections (Reference Only)

The Design Report should contain the following elements listed below. Note the (X-Y pg) indicates the amount of written text that should be in each section (not including tables, pictures, equations etc.).

Due with Final Report (Sections 1-12, Appendices A,B,C, and D):

Be sure to also make changes to report sections from DR1/DR2 (IN BLUE) to fix issues that were commented on by your GSI.

Section 9: Motor Controls (0.5-1 pg)

Introduce motor controls in general. Some questions you want to answer are:

- Why are motor controls needed (why can’t we just instruct the motor to move a certain number of encoder counts?)

- What sensors and electronics are used for motor control in this project?

- How does the quality of controls affect the outcome of important measured quantities in the project?

- Discuss PID and PWM control and the electronic components used for their execution.

- Discuss the capabilities and limitations of the proximity sensors. Briefly describe how the sensors function. Discuss noise in readings and limitations in sensor sensitivity.

Motor control is essential in this project because precise positioning of the flashlight determines the system’s ability to knock back targets in time. Simply commanding the motor to rotate a certain number of encoder counts is unreliable due to friction, mechanical backlash or the variation of motor voltage. Without feedback, these uncertainties will accumulate and degrade our system’s performance.

Our project uses DC permanent magnet motor with an integrated quadrature encoder for feedback. A limit switch is used for the startup calibration to define an initial position. Four analog proximity sensors, which are used for measuring the relative distance of each zombie, are mounted on the back wall of the playing field and connected to the Arduino board. The photosensors serve as verification sensors that detect whether the flashlight accurately illuminates the target, enabling the zombies to reverse the moving direction.

The quality of controls directly affects the system’s response time, positioning accuracy (steady-state error), and overall reliability. Poor control can lead to overshoot, delays, or missed targets, causing unwanted behaviour and low performance scores.

A PID control is implemented in Arduino to control the motor. The encoder provides position feedback, and the PID loop calculates the motor voltage required to minimize the error between the desired and actual positions. Proper tuning the PID control parameters minimizes overshoot and settling time while preventing oscillations. The motor voltage is implemented using PWM, which adjusts the motor voltage by rapidly switching it on and off with varying duty cycles. An H-bridge is used to control the motor’s direction and apply the PWM signal.

The proximity sensors are analog infrared distance sensors. They measure the reflected infrared light from the targets and output a voltage that decreases with increasing distance. However, these readings can be noisy due to environmental factors like ambient light. The effective range is limited, and sensitivity drops off beyond a certain distance, which constrains where accurate distance measurement can be made.

Appendix D: Wiring Diagram, Arduino Code Changes

Arduino Code: Include your Arduino code with sufficient comments, in particular where changes were made. Include line numbers so that your code can be easily referenced in the report.

Discuss changes made to the Arduino code, why, and how the changes were determined. Provide clear references to your code and/or the original code if necessary.

In our code, we define the values of:

- Limit for each proximity sensor when zombie is closest to photosensor/proximity sensor (line 25-32)

- The calibration voltage (line 107)

- The target position (line 108-111)

- The wait position (line 112)

- The PID parameters (line 128-130)

- The friction composite voltage (line 132) and supply voltage (line 131)

We also change some of the default values in the code:

- In line 55, we change the time to declare a zombie stopped 150ms less (default 250ms). We found that 100ms is enough for the same task the extra 150ms allows our linkage moves faster to the next target

- In line 115, we set the target band to 50. The default value is 10, but since the light range of our flashlight is big enough, we do not need to restrict this value to a small range. With larger target band, the robustness of our system increases, allowing the system to consider the current state to be within acceptable limits in the presence of noise and small fluctuations, thus avoiding frequent triggering of control actions due to overly aggressive limits.

- In line 85, we shorten the activate time since we found we don’t 350ms to activate a target. This gives the linkage more time to move to the next target.

Changed Lines in the Code:

……

022: //** Proximity Sensors or Potentiometer: **//

023: // CONSTANTS:

024: // Definition of proximity sensor Limits for each target:

025: const int PROXIMITYSENSE1_MAX = 612;     // [proximity sensor counts] Value of prox sensor 1 when zombie is closest to photosensor

026: const int PROXIMITYSENSE1_MIN = 97;     // [proximity sensor counts] Value of prox sensor 1 when zombie is closest to prox sensor

027: const int PROXIMITYSENSE2_MAX = 655;     // [proximity sensor counts] Value of prox sensor 2 when zombie is closest to photosensor

028: const int PROXIMITYSENSE2_MIN = 133;     // [proximity sensor counts] Value of prox sensor 2 when zombie is closest to prox sensor

029: const int PROXIMITYSENSE3_MAX = 620;     // [proximity sensor counts] Value of prox sensor 3 when zombie is closest to photosensor

030: const int PROXIMITYSENSE3_MIN = 146;     // [proximity sensor counts] Value of prox sensor 3 when zombie is closest to prox sensor

031: const int PROXIMITYSENSE4_MAX = 618;     // [proximity sensor counts] Value of prox sensor 4 when zombie is closest to photosensor

032: const int PROXIMITYSENSE4_MIN = 93;     // [proximity sensor counts] Value of prox sensor 4 when zombie is closest to prox sensor

……

052: // VARIABLES:

……

055: int stopTimeout = 100;                // [millis] time needed to declare a zombie stopped default 250ms. We change this time 150ms less since 100ms is enough to declare a zombie stopped, and makes our system move faster.

……

085: const int targetActivateTime = 340;   // time to activate a target (default is 350ms)

……

104: //** High-level behavior of the controller:  **//

105: // CONSTANTS:

106: // Target positions:

107: const int CALIBRATION_VOLTAGE  = -4; // [Volt] Motor voltage used during the calibration process

108: const int TARGET_1_POSITION    = 0; // [encoder counts] Motor position corresponding to first target

109: const int TARGET_2_POSITION    = 125; // [encoder counts] Motor position corresponding to second target

110: const int TARGET_3_POSITION    = 246; // [encoder counts] Motor position corresponding to third target

111: const int TARGET_4_POSITION    = 560; // [encoder counts] Motor position corresponding to fourth target

112: const int WAIT_POSITION        = TARGET_2_POSITION; // [encoder counts] Motor position corresponding to a wait position (when no targets are active)

113: const int LOWER_BOUND          = TARGET_1_POSITION; // [encoder counts] Position of the left end stop

114: const int UPPER_BOUND          = TARGET_4_POSITION; // [encoder counts] Position of the right end stop

115: const int TARGET_BAND          = 50; // [encoder counts] "Close enough" range when moving towards a target.

……

123: // VARIABLES:

124: //unsigned long startWaitTime; // [microseconds] System clock value

125:

126: //** PID Controller  **//

127: // CONSTANTS:

128: const float KP             =          0.08;               // [Volt / encoder counts] P-Gain

129: const float KI             =          0.02;               // [Volt / (encoder counts * seconds)] I-Gain

130: const float KD             =          0.008;               // [Volt * seconds / encoder counts] D-Gain

131: const float SUPPLY_VOLTAGE =          9;               // [Volt] Supply voltage at the HBridge

132: const float FRICTION_COMP_VOLTAGE =   2.75;               // [Volt] Voltage needed to overcome friction

……

Full Code:

001: // ME350 Zombie Defense Sketch - Version 05.1 by Spencer Dickhudt

002: // updated 12-5-2024

003: //

004: //

005:

006: //////////////////////////////////////////////

007: // DEFINE CONSTANTS AND GLOBAL VARIABLES:   //

008: //////////////////////////////////////////////

009:

010: //** State Machine: **//

011: // CONSTANTS:

012: // Definition of states in the state machine

013: const int CALIBRATE                = 1;

014: const int CHOOSE_ACTIVE_TARGET     = 2;

015: const int MOVE_TO_TARGET           = 3;

016:

017: // VARIABLES:

018: // Global variable that keeps track of the state:

019: // Start the state machine in calibration state:

020: int state = CALIBRATE;

021:

022: //** Proximity Sensors or Potentiometer: **//

023: // CONSTANTS:

024: // Definition of proximity sensor Limits for each target:

025: const int PROXIMITYSENSE1_MAX = 612;     // [proximity sensor counts] Value of prox sensor 1 when zombie is closest to photosensor

026: const int PROXIMITYSENSE1_MIN = 97;     // [proximity sensor counts] Value of prox sensor 1 when zombie is closest to prox sensor

027: const int PROXIMITYSENSE2_MAX = 655;     // [proximity sensor counts] Value of prox sensor 2 when zombie is closest to photosensor

028: const int PROXIMITYSENSE2_MIN = 133;     // [proximity sensor counts] Value of prox sensor 2 when zombie is closest to prox sensor

029: const int PROXIMITYSENSE3_MAX = 620;     // [proximity sensor counts] Value of prox sensor 3 when zombie is closest to photosensor

030: const int PROXIMITYSENSE3_MIN = 146;     // [proximity sensor counts] Value of prox sensor 3 when zombie is closest to prox sensor

031: const int PROXIMITYSENSE4_MAX = 618;     // [proximity sensor counts] Value of prox sensor 4 when zombie is closest to photosensor

032: const int PROXIMITYSENSE4_MIN = 93;     // [proximity sensor counts] Value of prox sensor 4 when zombie is closest to prox sensor

033:

034: const int ProxRange[4][2] = {{PROXIMITYSENSE1_MAX, PROXIMITYSENSE1_MIN},

035:                              {PROXIMITYSENSE2_MAX, PROXIMITYSENSE2_MIN},

036:                              {PROXIMITYSENSE3_MAX, PROXIMITYSENSE3_MIN},

037:                              {PROXIMITYSENSE4_MAX, PROXIMITYSENSE4_MIN}}; // Array holding prox sensor bounds to improve code readability

038:

039: // Definition of target variables for array indexing

040: const int TARGET1  = 0;   // For indexing into activeTargets array, activeTargets[0] corresponds to Target 1

041: const int TARGET2  = 1;   // For indexing into activeTargets array, activeTargets[1] corresponds to Target 2

042: const int TARGET3  = 2;   // For indexing into activeTargets array, activeTargets[2] corresponds to Target 3

043: const int TARGET4  = 3;   // For indexing into activeTargets array, activeTargets[3] corresponds to Target 4

044:

045: const int targetArr[4] = { TARGET1, TARGET2, TARGET3, TARGET4 };  // List of target indices to reduce redundant code

046:

047: // Direction variables to represent the movement of each zombie

048: const int FORWARD = 1;      // representing the zombie moving towards the photosensor

049: const int BACKWARD = -1;    // representing the zombie moving away from the photosensor

050: const int STOPPED = 0;      // representing the zombie stalled

051:

052: // VARIABLES:

053: // Variables for updating sensor readings

054: const float alpha = 0.925;            // filter parameter used to weight current and past readings

055: int stopTimeout = 100;                // [millis] time needed to declare a zombie stopped default 250ms

056: int noiseLimit = 8;                   // average spread of proximity readings for a stopped zombie

057: const int lowerNoiseLimit = 5;        // noise limit for near the photosensors

058: const int upperNoiseLimit = 8;        // noise limit for near the prox sensors

059: const int noiseThreshold = 225;       // prox sensor reading at which noise limit switches from upper to lower

060:

061: // A data structure to hold proximity sensor information:

062: struct Sensor{

063:   float currVal;                    // the most recent proximity sensor reading

064:   float prevVal;                    // the last update of proximity sensor reading

065:   unsigned long prevChangeTime;     // [millis] the time at which prevVal last changed

066:   int pin;                          // the input pin of the desired prox sensor

067:   int direction;                    // the direction at which the zombie on the rail is traveling

068:   int forwardCount;                 // number of readings which indicate the zombie is moving forward

069:   int backwardCount;                // number of readings which indicate the zombie is moving backward

070: };

071:

072: Sensor ProxSensors[4];          // An array which holds Sensor objects, representing each of the prox sensors

073: int potValue = 0;               // Used to hold the reading of the potentiometer

074:

075: int activeTargetIndex = -1;     // index (in the activeTargets array) of the closest target to the plants

076: int activeTargetPosition = -1;  // [encoder counts] Encoder count of the active target the linkage is moving toward

077:

078: int minIndex = -1;              // tracking index of the zombie who is closest to the plants

079: float closestZombieDist = 2;    // [percentage] records the lowest percentage of rail remaining for any of the zombies

080: int idx = -1;                   // index of target, based on targetArr

081:

082: bool WAIT_POS = true;           // tracking if the linkage is moving to the wait position

083:

084: unsigned long arrivalTime;            // timer for tracking a wait period upon reaching a desired position

085: const int targetActivateTime = 340;   // time to activate a target (should be 350)

086: float Zombies[4];                     // An array to hold information on locations of all zombies in play

087:

088: // float sampleTime = 1.5             // sample time in ms

089:

090: //** Computation of position and velocity: **//

091: // CONSTANTS:

092: // Settings for velocity computation:

093: const int MIN_VEL_COMP_COUNT = 2;     // [encoder counts] Minimal change in motor position that must happen between two velocity measurements

094: const long MIN_VEL_COMP_TIME = 10000; // [microseconds] Minimal time that must pass between two velocity measurements

095: // VARIABLES:

096: volatile int motorPosition = 0; // [encoder counts] Current motor position (Declared 'volatile', since it is updated in a function called by interrupts)

097: volatile int encoderStatus = 0; // [binary] Past and Current A&B values of the encoder  (Declared 'volatile', since it is updated in a function called by interrupts)

098: // The rightmost two bits of encoderStatus will store the encoder values from the current iteration (A and B).

099: // The two bits to the left of those will store the encoder values from the previous iteration (A_old and B_old).

100: float motorVelocity        = 0; // [encoder counts / seconds] Current motor velocity

101: int previousMotorPosition  = 0; // [encoder counts] Motor position the last time a velocity was computed

102: long previousVelCompTime   = 0; // [microseconds] System clock value the last time a velocity was computed

103:

104: //** High-level behavior of the controller:  **//

105: // CONSTANTS:

106: // Target positions:

107: const int CALIBRATION_VOLTAGE  = -4; // [Volt] Motor voltage used during the calibration process

108: const int TARGET_1_POSITION    = 0; // [encoder counts] Motor position corresponding to first target

109: const int TARGET_2_POSITION    = 125; // [encoder counts] Motor position corresponding to second target

110: const int TARGET_3_POSITION    = 246; // [encoder counts] Motor position corresponding to third target

111: const int TARGET_4_POSITION    = 560; // [encoder counts] Motor position corresponding to fourth target

112: const int WAIT_POSITION        = TARGET_2_POSITION; // [encoder counts] Motor position corresponding to a wait position (when no targets are active)

113: const int LOWER_BOUND          = TARGET_1_POSITION; // [encoder counts] Position of the left end stop

114: const int UPPER_BOUND          = TARGET_4_POSITION; // [encoder counts] Position of the right end stop

115: const int TARGET_BAND          = 50; // [encoder counts] "Close enough" range when moving towards a target.

116:

117: // List of target positions to reduce redundant code

118: const int targetPos[4] = {TARGET_1_POSITION, TARGET_2_POSITION, TARGET_3_POSITION, TARGET_4_POSITION};

119:

120: // Timing:

121: //const long  WAIT_TIME          = 0; // [microseconds] Time waiting for the target to drop.

122:                                       // TBD - implement a timer so if the target doesn't drop, the linkage moves to a different target

123: // VARIABLES:

124: //unsigned long startWaitTime; // [microseconds] System clock value

125:

126: //** PID Controller  **//

127: // CONSTANTS:

128: const float KP             =          0.08;               // [Volt / encoder counts] P-Gain

129: const float KI             =          0.02;               // [Volt / (encoder counts * seconds)] I-Gain

130: const float KD             =          0.008;               // [Volt * seconds / encoder counts] D-Gain

131: const float SUPPLY_VOLTAGE =          9;               // [Volt] Supply voltage at the HBridge

132: const float FRICTION_COMP_VOLTAGE =   2.75;               // [Volt] Voltage needed to overcome friction

133: // VARIABLES:

134: int desiredPosition  = 0; // [encoder counts] desired motor position

135: float positionError  = 0; // [encoder counts] Position error

136: float integralError  = 0; // [encoder counts * seconds] Integrated position error

137: float velocityError  = 0; // [encoder counts / seconds] Velocity error

138: float desiredVoltage = 0; // [Volt] Desired motor voltage

139: int   motorCommand   = 0; // [0-255] PWM signal sent to the motor

140: unsigned long executionDuration = 0; // [microseconds] Time between this and the previous loop execution.  Variable used for integrals and derivatives

141: unsigned long lastExecutionTime = 0; // [microseconds] System clock value at the moment the loop was started the last time

142:

143: //** Pin assignment: **//

144: // CONSTANTS:

145: const int PIN_NR_ENCODER_A        = 2;  // Never change these, since the interrupts are attached to pins 2 and 3

146: const int PIN_NR_ENCODER_B        = 3;  // Never change these, since the interrupts are attached to pins 2 and 3

147: const int PIN_NR_ON_OFF_SWITCH    = 5;  // Connected to toggle switch (turns mechanism on and off)

148: const int PIN_NRL_LIMIT_SWITCH    = 8;  // Connected to limit switch (mechanism calibration)

149: const int PIN_NR_PWM_OUTPUT       = 11; // Connected to H Bridge (controls motor speed)

150: const int PIN_NR_PWM_DIRECTION_1  = 12; // Connected to H Bridge (controls motor direction)

151: const int PIN_NR_PWM_DIRECTION_2  = 13;  // Connected to H Bridge (controls motor direction)

152: const int PIN_PROXSENSE1          = A0; // Connected to proximity sensor 1

153: const int PIN_PROXSENSE2          = A1; // Connected to proximity sensor 2

154: const int PIN_PROXSENSE3          = A2; // Connected to proximity sensor 3

155: const int PIN_PROXSENSE4          = A3; // Connected to proximity sensor 4

156: const int PIN_POTENTIOMETER       = A4; // Connected to potentiometer used to test target positions

157:

158:

159: // Add this line of code if you want to use two limit switches; const int PIN_NRL_LIMIT_SWITCH_2  = 11

160: // ^KEEP IN MIND THAT YOU HAVE TO ADD CODE DOWNSTREAM (FOR EXAMPLE YOU NEED TO ADD THIS VARIABLE IN THE DECLARATION SECTION

161:

162: // End of CONSTANTS AND GLOBAL VARIABLES

163:

164:

165: //////////////////////////////////////////////////////////////////////////////////////////

166: // The setup() function is called when a sketch starts. Use it to initialize variables, //

167: // pin modes, start using libraries, etc. The setup function will only run once, after  //

168: // each powerup or reset of the Arduino board:                                          //

169: //////////////////////////////////////////////////////////////////////////////////////////

170: void setup() {

171:   // Declare which digital pins are inputs and which are outputs:

172:   pinMode(PIN_NR_ENCODER_A,        INPUT_PULLUP);

173:   pinMode(PIN_NR_ENCODER_B,        INPUT_PULLUP);

174:   pinMode(PIN_NR_ON_OFF_SWITCH,    INPUT);

175:   pinMode(PIN_NRL_LIMIT_SWITCH,    INPUT);

176:   pinMode(PIN_PROXSENSE1,          INPUT);

177:   pinMode(PIN_PROXSENSE2,          INPUT);

178:   pinMode(PIN_PROXSENSE3,          INPUT);

179:   pinMode(PIN_PROXSENSE4,          INPUT);

180:   pinMode(PIN_NR_PWM_OUTPUT,       OUTPUT);

181:   pinMode(PIN_NR_PWM_DIRECTION_1,  OUTPUT);

182:   pinMode(PIN_NR_PWM_DIRECTION_2,  OUTPUT);

183:   pinMode(PIN_POTENTIOMETER,       INPUT);

184:

185:   // Turn on the pullup resistors on the encoder channels

186:   digitalWrite(PIN_NR_ENCODER_A, HIGH);

187:   digitalWrite(PIN_NR_ENCODER_B, HIGH);

188:

189:   // Activate interrupt for encoder pins.

190:   // If either of the two pins changes, the function 'updateMotorPosition' is called:

191:   attachInterrupt(0, updateMotorPosition, CHANGE);  // Interrupt 0 is always attached to digital pin 2

192:   attachInterrupt(1, updateMotorPosition, CHANGE);  // Interrupt 1 is always attached to digital pin 3

193:

194:   // Begin serial communication for monitoring.

195:   Serial.begin(115200);

196:   Serial.println("Start Executing Program.");

197:

198:   // Initialize sensor values

199:   ProxSensors[TARGET1].prevVal = analogRead(PIN_PROXSENSE1);

200:   ProxSensors[TARGET2].prevVal = analogRead(PIN_PROXSENSE2);

201:   ProxSensors[TARGET3].prevVal = analogRead(PIN_PROXSENSE3);

202:   ProxSensors[TARGET4].prevVal = analogRead(PIN_PROXSENSE4);

203:

204:   // Initialize prox sensor pins

205:   ProxSensors[TARGET1].pin = PIN_PROXSENSE1;

206:   ProxSensors[TARGET2].pin = PIN_PROXSENSE2;

207:   ProxSensors[TARGET3].pin = PIN_PROXSENSE3;

208:   ProxSensors[TARGET4].pin = PIN_PROXSENSE4;

209:

210:   // Set initial output to the motor to 0

211:   analogWrite(PIN_NR_PWM_OUTPUT, 0);

212: }

213: // End of function setup()

214:

215:

216: ////////////////////////////////////////////////////////////////////////////////////////////////

217: // After going through the setup() function, which initializes and sets the initial values,   //

218: // the loop() function does precisely what its name suggests, and loops consecutively,        //

219: // allowing your program to sense and respond. Use it to actively control the Arduino board.  //

220: ////////////////////////////////////////////////////////////////////////////////////////////////

221: void loop() {

222:   // Determine the duration it took to execute the last loop. This time is used

223:   // for integration and for monitoring the loop time via the serial monitor.

224:   executionDuration = micros() - lastExecutionTime;

225:   lastExecutionTime = micros();

226:

227:   // Speed Computation:

228:   if ((abs(motorPosition - previousMotorPosition) > MIN_VEL_COMP_COUNT) || (micros() - previousVelCompTime) > MIN_VEL_COMP_TIME){

229:     // If at least a minimum time interval has elapsed or

230:     // the motor has travelled through at least a minimum angle ...

231:     // .. compute a new value for speed:

232:     // (speed = delta angle [encoder counts] divided by delta time [seconds])

233:     motorVelocity = (double)(motorPosition - previousMotorPosition) * 1000000 /

234:                             (micros() - previousVelCompTime);

235:     // Remember this encoder count and time for the next iteration:

236:     previousMotorPosition = motorPosition;

237:     previousVelCompTime   = micros();

238:   }

239:   // *****************************************************************************//

240:   // We first update the sensors so our direction information stays accurate

241:

242:   // Loop through each of the sensors

243:   for (int i = 0; i < sizeof(targetArr)/sizeof(int); i++) {

244:

245:     idx = targetArr[i];

246:     // Updates the current sensor reading using a low pass filter

247:     ProxSensors[idx].currVal = alpha * ProxSensors[idx].currVal + (1 - alpha) * analogRead(ProxSensors[idx].pin);

248:

249:     // Adjusts the noise Limit based on distance to sensor (since values are not linear)

250:     if (ProxSensors[idx].currVal >= noiseThreshold) {

251:       noiseLimit = upperNoiseLimit;

252:     } else {

253:       noiseLimit = lowerNoiseLimit;

254:     }

255:

256:     // Check if sensor is stopped

257:     if (abs(ProxSensors[idx].currVal - ProxSensors[idx].prevVal) < noiseLimit) {

258:       // if not enough time has passed to declare stopped or already stopped continue loop

259:       if (millis() - ProxSensors[idx].prevChangeTime < stopTimeout || ProxSensors[idx].direction == STOPPED) {

260:         ProxSensors[i].forwardCount = 0;

261:         ProxSensors[i].backwardCount = 0;

262:         continue;

263:       } else {

264:       // the prox sensor has been stopped for long enough that we know it is stopped

265:         ProxSensors[idx].direction = STOPPED;

266:       }

267:     } else if (ProxSensors[idx].currVal - ProxSensors[idx].prevVal < 0){

268:       // Target is moving forward, as readings decrease moving further away from the sensor

269:       ProxSensors[i].forwardCount += 1;

270:       // Target is moving forward

271:       if (ProxSensors[i].forwardCount > 3){

272:         ProxSensors[i].direction = FORWARD;

273:

274:         ProxSensors[i].prevVal = ProxSensors[i].currVal;

275:         ProxSensors[i].prevChangeTime = millis();

276:       }

277:     } else {

278:       // Target must be moving backward

279:       ProxSensors[i].backwardCount += 1;

280:       // Target is moving forward

281:       if (ProxSensors[i].backwardCount > 3){

282:         ProxSensors[i].direction = BACKWARD;

283:

284:         ProxSensors[i].prevVal = ProxSensors[i].currVal;

285:         ProxSensors[i].prevChangeTime = millis();

286:       }

287:     }

288:   }

289:   // *****************************************************************************//

290:

291:   //******************************************************************************//

292:   // The state machine:

293:   switch (state) {

294:     //****************************************************************************//

295:     // In the CALIBRATE state, we move the mechanism to a position outside of the

296:     // work space (towards the limit switch).  Once the limit switch is on and

297:     // the motor has stopped turning, we know that we are against the end stop

298:     case CALIBRATE:

299:       // We don't have to do anything here since this state is only used to set

300:       // a fixed output voltage.  This happens further below.

301:

302:       // Decide what to do next:

303:       if (digitalRead(PIN_NRL_LIMIT_SWITCH)==HIGH && motorVelocity==0) {

304:         // We reached the endstop.  Update the motor position to the limit:

305:         // (NOTE: If the limit switch is on the right, this must be UPPER_BOUND)

306:         motorPosition = LOWER_BOUND;

307:         // Reset the error integrator:

308:         integralError = 0;

309:         // Calibration is finalized. Transition into DETERMINE_ACTIVE_TARGETS state

310:         Serial.println("State transition from CALIBRATE to CHOOSE_ACTIVE_TARGET");

311:         state = CHOOSE_ACTIVE_TARGET;

312:       }

313:

314:       // Otherwise we continue calibrating

315:       break;

316:

317:     //****************************************************************************//

318:     // In the DETERMINE_ACTIVE_TARGETS state, we use the most recent reading of the photosensors to

319:     // calculate the location of each zombie.

320:     // We then select an active target to move toward based on which zombie is closest to the plants.

321:     // We default to WAIT_POSITION if no targets are active.

322:     case CHOOSE_ACTIVE_TARGET:

323:

324:       // Decide what zombie is closest to the photosensor

325:       // Current closest zombie is out of bounds

326:       minIndex = -1;

327:       // Set distance to 2 to be above any possible percentage values + noise

328:       closestZombieDist = 2;

329:

330:       for (int i = 0; i < sizeof(targetArr)/sizeof(int); i++) {

331:         // Use the target array for the proper indexes into other arrays

332:         idx = targetArr[i];

333:

334:         // Calculate the distance to the front of the rail by taking a percentage of how much rail

335:         // the zombie has left to travel.

336:         Zombies[idx] = (ProxSensors[idx].currVal - ProxRange[idx][1]) / (ProxRange[idx][0] - ProxRange[idx][1]);

337:

338:         // Check to see if the zombie is traveling forward and it is closer than the previous zombie

339:         if (ProxSensors[idx].direction == FORWARD && Zombies[idx] < closestZombieDist) {

340:           // update the newest minimum value

341:           closestZombieDist = Zombies[idx];

342:           // update the index of the active target (in the targetArr array)

343:           minIndex = i;

344:         }

345:       }

346:

347:       // if the min index points to a valid target, move to that target

348:       if (minIndex >= 0) {

349:         activeTargetIndex = targetArr[minIndex];

350:         activeTargetPosition = targetPos[activeTargetIndex];

351:         WAIT_POS = false;

352:         // Serial.println("Setting position to something other than wait");

353:       } else {

354:         activeTargetPosition = WAIT_POSITION;

355:         WAIT_POS = true;

356:         // Serial.println("Setting position to wait position");

357:       }

358:

359:       state = MOVE_TO_TARGET;

360:       // Serial.println("Switching state to MOVE_TO_TARGET");

361:

362:       // Otherwise, we stay in DETERMINE_ACTIVE_TARGETS

363:       break;

364:

365:

366:     //****************************************************************************//

367:     // In the MOVE_TO_TARGET state, we select an active target and move toward it, or

368:     // move toward Target 3 (a default position) if there is no active target

369:     case MOVE_TO_TARGET:

370:       // Serial.println("Inside MOVE_TO_TARGET");

371:       desiredPosition = activeTargetPosition;

372:

373:       if (motorPosition <= activeTargetPosition + TARGET_BAND && motorPosition >= activeTargetPosition - TARGET_BAND) {

374:

375:         if (millis() - arrivalTime > targetActivateTime || WAIT_POS){

376:           state = CHOOSE_ACTIVE_TARGET;

377:           Serial.println(millis() - arrivalTime > targetActivateTime);

378:         }

379:       } else {

380:         arrivalTime = millis();

381:       }

382:

383:       break;

384:

385:     //****************************************************************************//

386:     //****************************************************************************//

387:     // We should never reach the next bit of code, which would mean that the state

388:     // we are currently in doesn't exist.  So if it happens, throw an error and

389:     // stop the program:

390:     default:

391:       Serial.println("Statemachine reached at state that it cannot handle.  ABORT!!!!");

392:       Serial.print("Found the following unknown state: ");

393:       Serial.println(state);

394:       while (1); // infinite loop to halt the program

395:     break;

396:   }

397:   // End of the state machine.

398:   //******************************************************************************//

399:

400:   //******************************************************************************//

401:   // Recalibrate if we are in the leftmost position

402:   if (digitalRead(PIN_NRL_LIMIT_SWITCH)==HIGH && motorVelocity==0) {

403:         // We reached the endstop.  Update the motor position to the limit:

404:         // (NOTE: If the limit switch is on the right, this must be UPPER_BOUND)

405:         motorPosition = LOWER_BOUND;

406:         // Reset the error integrator:

407:         integralError = 0;

408:         Serial.println("Limit Switch hit");

409:   }

410:

411:

412:   //******************************************************************************//

413:   // Position Controller

414:   if (digitalRead(PIN_NR_ON_OFF_SWITCH)==HIGH) {

415:     // If the toggle switch is on, run the controller:

416:

417:     //** PID control: **//

418:     // Compute the position error [encoder counts]

419:     positionError = desiredPosition - motorPosition;

420:     // Compute the integral of the position error  [encoder counts * seconds]

421:     integralError = integralError + positionError * (float)(executionDuration) / 1000000;

422:     // Compute the velocity error (desired velocity is 0) [encoder counts / seconds]

423:     velocityError = 0 - motorVelocity;

424:     // This is the actual controller function that uses the error in

425:     // position and velocity and the integrated error and computes a

426:     // desired voltage that should be sent to the motor:

427:     desiredVoltage = KP * positionError +

428:                      KI * integralError +

429:                      KD * velocityError;

430:

431:     //** Feedforward terms: **//

432:     // Compensate for friction.  That is, if we now the direction of

433:     // desired motion, add a base command that helps with moving in this

434:     // direction:

435:     if (positionError < -5) {

436:       desiredVoltage = desiredVoltage - FRICTION_COMP_VOLTAGE;

437:     }

438:     if (positionError > +5) {

439:       desiredVoltage = desiredVoltage + FRICTION_COMP_VOLTAGE;

440:     }

441:

442:     // Anti-Wind-Up

443:     if (abs(desiredVoltage)>SUPPLY_VOLTAGE) {

444:       // If we are already saturating our output voltage, it does not make

445:       // sense to keep integrating the error (and thus ask for even higher

446:       // and higher output voltages).  Instead, stop the integrator if the

447:       // output saturates. We do this by reversing the summation at the

448:       // beginning of this function block:

449:       integralError = integralError - positionError * (float)(executionDuration) / 1000000;

450:     }

451:     // End of 'if(onOffSwitch==HIGH)'

452:

453:     // Override the computed voltage during calibration.  In this state, we simply apply a

454:     // fixed voltage to move against one of the end-stops.

455:     if (state==CALIBRATE) {

456:       // add calibration code here

457:       desiredVoltage = CALIBRATION_VOLTAGE;

458:     }

459:   } else {

460:     // Otherwise, the toggle switch is off, so do not run the controller,

461:     // stop the motor...

462:     desiredVoltage = 0;

463:     // .. and reset the integrator of the error:

464:     integralError = 0;

465:     // Produce some debugging output:

466:     Serial.println("The toggle switch is off.  Motor Stopped.");

467:   }

468:   // End of  else onOffSwitch==HIGH

469:

470:   //** Send signal to motor **//

471:   // Convert from voltage to PWM cycle:

472:   motorCommand = int(abs(desiredVoltage * 255 / SUPPLY_VOLTAGE));

473:   // Clip values larger than 255

474:   if (motorCommand > 255) {

475:     motorCommand = 255;

476:   }

477:   // Send motor signals out

478:   analogWrite(PIN_NR_PWM_OUTPUT, motorCommand);

479:   // Determine rotation direction

480:   if (desiredVoltage >= 0) {

481:     // If voltage is positive ...

482:     // ... turn forward

483:     digitalWrite(PIN_NR_PWM_DIRECTION_1,LOW);  // rotate forward

484:     digitalWrite(PIN_NR_PWM_DIRECTION_2,HIGH); // rotate forward

485:   } else {

486:     // ... otherwise turn backward:

487:     digitalWrite(PIN_NR_PWM_DIRECTION_1,HIGH); // rotate backward

488:     digitalWrite(PIN_NR_PWM_DIRECTION_2,LOW);  // rotate backward

489:   }

490:   // End of Position Controller

491:   //*********************************************************************//

492:

493:   // Print out current controller state to Serial Monitor.

494:   printStateToSerial();

495: }

496: // End of main loop

497: //***********************************************************************//

498:

499:

500: //////////////////////////////////////////////////////////////////////

501: // This is a function to update the encoder count in the Arduino.   //

502: // It is called via an interrupt whenever the value on encoder      //

503: // channel A or B changes.                                          //

504: //////////////////////////////////////////////////////////////////////

505: void updateMotorPosition() {

506:   // Bitwise shift left by one bit, to make room for a bit of new data:

507:   encoderStatus <<= 1;

508:   // Use a compound bitwise OR operator (|=) to read the A channel of the encoder (pin 2)

509:   // and put that value into the rightmost bit of encoderStatus:

510:   encoderStatus |= digitalRead(2);

511:   // Bitwise shift left by one bit, to make room for a bit of new data:

512:   encoderStatus <<= 1;

513:   // Use a compound bitwise OR operator  (|=) to read the B channel of the encoder (pin 3)

514:   // and put that value into the rightmost bit of encoderStatus:

515:   encoderStatus |= digitalRead(3);

516:   // encoderStatus is truncated to only contain the rightmost 4 bits by  using a

517:   // bitwise AND operator on mstatus and 15(=1111):

518:   encoderStatus &= 15;

519:   if (encoderStatus==2 || encoderStatus==4 || encoderStatus==11 || encoderStatus==13) {

520:     // the encoder status matches a bit pattern that requires counting up by one

521:     motorPosition++;         // increase the encoder count by one

522:   }

523:   else if (encoderStatus == 1 || encoderStatus == 7 || encoderStatus == 8 || encoderStatus == 14) {

524:     // the encoder status does not match a bit pattern that requires counting up by one.

525:     // Since this function is only called if something has changed, we have to count downwards

526:     motorPosition--;         // decrease the encoder count by one

527:   }

528: }

529: // End of function updateMotorPosition()

530:

531:

532: //////////////////////////////////////////////////////////////////////

533: // This function sends a status of the controller to the serial     //

534: // monitor.  Each character will take 85 microseconds to send, so   //

535: // be selective in what you write out:                              //

536: //////////////////////////////////////////////////////////////////////

537: void printStateToSerial() {

538:   //*********************************************************************//

539:   // Send a status of the controller to the serial monitor.

540:   // Each character will take 85 microseconds to send, so be selective

541:   // in what you write out:

542:

543:   //Serial.print("State Number:  [CALIBRATE = 1; DETERMINE_ACTIVE_TARGETS = 2; MOVE_TO_TARGET = 3]: ");

544:   Serial.print("State#: ");

545:   Serial.print(state);

546:

547:   //Serial.print("Power switch [on/off]: ");

548:   //Serial.print("  PWR: ");

549:   //Serial.print(digitalRead(PIN_NR_ON_OFF_SWITCH));

550:

551:   //Serial.print("      Motor Position [encoder counts]: ");

552:   Serial.print("  MP: ");

553:   Serial.print(motorPosition);

554:

555:   //Serial.print("      Motor Velocity [encoder counts / seconds]: ");

556:   Serial.print("  MV: ");

557:   Serial.print(motorVelocity);

558:

559:   //Serial.print("      Encoder Status [4 bit value]: ");

560:   //Serial.print("  ES: ");

561:   //Serial.print(encoderStatus);

562:

563:   //Serial.print("      Target Position [encoder counts]: ");

564:   Serial.print("  DP: ");

565:   Serial.print(desiredPosition);

566:

567:   // //Serial.print("      Position Error [encoder counts]: ");

568:   // Serial.print("  PE: ");

569:   // Serial.print(positionError);

570:

571:   // //Serial.print("      Integrated Error [encoder counts * seconds]: ");

572:   // Serial.print("  IE: ");

573:   // Serial.print(integralError);

574:

575:   // //Serial.print("      Velocity Error [encoder counts / seconds]: ");

576:   // Serial.print("  VE: ");

577:   // Serial.print(velocityError);

578:

579:   //Serial.print("      Desired Output Voltage [Volt]: ");

580:   Serial.print("  DV: ");

581:   Serial.print(desiredVoltage);

582:

583:   //Serial.print("      Motor Command [0-255]: ");

584:   //Serial.print("  MC: ");

585:   //Serial.print(motorCommand);

586:

587:   //Serial.print("      Execution Duration [microseconds]: ");

588:   //Serial.print("  ED: ");

589:   //Serial.print(executionDuration);

590:

591:   //Serial.print("      Zombie Location [% of rail left to travel]: ");

592:   Serial.print("  ZL: ");

593:   for (int i = 0; i < sizeof(Zombies)/sizeof(float); i++) {

594:     Serial.print(Zombies[i]);

595:     Serial.print(" ");

596:   }

597:

598:   //Serial.print("      Proxsensor Direction[STOPPED or FORWARD or BACKWARD]: ");

599:   Serial.print("  DR: ");

600:   for (int i = 0; i < sizeof(Zombies)/sizeof(float); i++) {

601:     Serial.print(ProxSensors[i].direction);

602:     Serial.print(" ");

603:   }

604:

605:   // ALWAYS END WITH A NEWLINE.  SERIAL MONITOR WILL CRASH IF NOT

606:   Serial.println(); // new line

607: }

608: // End of Serial Out

609:

610: //////////////////////////////////////////////////////////////////////

611: // This function returns the average of the integer array           //

612: // pointed to by array_ptr and of length len.                       //

613: //////////////////////////////////////////////////////////////////////

614: float average (int * array_ptr, int len) {

615:   long sum = 0L;

616:   for (int i = 0; i < len; i++) {

617:     sum += array_ptr[i];

618:   }

619:   return ((float) sum)/len;

620: }

621: // End of average